; O2init.doc

; This is the document file and the sample data file for o2init.sys

;  and o2init.com.

; This file can be executed as a demo, try 'o2init o2init.doc'

; Warning: it writes to com1 and other devices!

# Usage: 'device=o2init.sys <datafile>' or 'o2init.com <datafile>'

# Commands are read from the datafile and acted on.  The data file

# is limited to 24kb.  If you want to do more than 24k (!), split

# the data into 2 files and run o2init twice.

; Comments are marked by either ; or #, and extend to the end of the line.

; All numbers are hexidecimal.

# The available commands are:

#

# PCIFIND vend dev func - searches for specified vendor:device:func, sets the found flag.

# PCIMORE vend dev func - searches for specified vendor:device:func, starts at last found device, sets the found flag.

# IBFIND port data - compares input port byte value to data, sets found flag.

# IWFIND port data - compares input port word value to data, sets found flag.

# IDFIND port data - compares input port dword value to data, sets found flag.

# PCICOMPAREB data - compares PCI byte from current device to data, sets found flag.

# PCICOMPAREW data - compares PCI word from current device to data, sets found flag.

# PCICOMPARED data - compares PCI dword from current device to data, sets found flag.

#

# PCIREADB reg - reads the byte at reg from the last found PCI device.

# PCIREADW reg - reads a word at reg from last PCIFIND device. Reg must be even.

# PCIREADD reg - reads a dword at reg.  Reg must be 0, 4, 6, 12, etc.

# PCIWRITEB reg data - writes byte to last found PCI device.

# PCIWRITEW reg data - writes word to PCI device. Reg must be even.

# PCIWRITED reg data - writes dword to PCI device. Reg must be 0, 4, 8, etc.

# PCIORB reg ordata - reads byte, ORs with data and writes byte to PCI device.

# PCIORW reg ordata - reads word, ORs with data and writes byte to PCI device.

# PCIORD reg ordata - reads dword, ORs with data and writes byte to PCI device.

# PCIANDB reg anddata - reads byte, ANDs with data and writes byte to PCI device.

# PCIANDW reg anddata - reads word, ANDs with data and writes byte to PCI device.

# PCIANDD reg anddata - reads dword, ANDs with data and writes byte to PCI device.

# PCIXORB reg anddata - reads byte, XORs with data and writes byte to PCI device.

# PCIXORW reg anddata - reads word, XORs with data and writes byte to PCI device.

# PCIXORD reg anddata - reads dword, XORs with data and writes byte to PCI device.

# Note: PCI regs are from 0 to FF

#

# IB port - inputs port byte value, displays.

# IW port - inputs port word value, displays.

# ID port - inputs port dword value, displays.

# OB port data - outputs port byte value.

# OW port data - outputs port word value.

# OD port data - outputs port dword value.

# ORB port data - inputs byte, ORs with data, outputs result.

# ORW port data - inputs word, ORs with data, outputs result.

# ORD port data - inputs dword, ORs with data, outputs result.

# ANDB port data - inputs byte, ANDs with data, outputs result.

# ANDW port data - inputs word, ANDs with data, outputs result.

# ANDD port data - inputs dword, ANDs with data, outputs result.

# XORB port data - inputs byte, XORs with data, outputs result.

# XORW port data - inputs word, XORs with data, outputs result.

# XORD port data - inputs dword, XORs with data, outputs result.

#

# QUIET - minimum screen output, shows finds and errors only.

# SHOW  - medimum screen output, shows finds, errors, and read/input results.

# VERBOSE - all screen output.

# PAUSE - stops and waits for a keypress, useful for debugging.

; The 'found' flag is powerful.  If a find fails, all commands will be

; ignored until a sucessful find happens.  Writes to a device can be

; skipped if the device is not found in the system.  A PCI find, compare,

; or an input find will set the 'found' flag.  A PCIfind FFFF FFFF will

; always be found, an inputfind from FFFF will always be found, and

; either will set the found flag TRUE.  When the program starts, the

; found file is TRUE.

# Commands before any FIND will always happen, but PCI commands will

# fail if there is not a preceeding PCI find.  PCI commands use BIOS

# calls.  If there is no PCI bios, strange things will happen on PCI

# functions.

# The startup default output flag is SHOW

orb 3fb 80              # set DLAB bit

ob 3f8 0C

ob 3f9 0                # set com1 to 9600 baud

andb 3fb 7F             # clear DLAB bit

ob 3f9 0

ob 3fC 03               # set handshake lines on

ib 3f8

OB 3f8 59               # send some serial data

OB 3f8 6F

OB 3f8 21

ob 3f8 0d

ob 3f8 0a

# output flag - use Quiet, Show, Verbose for minimum, medium, most output

verbose                 ; show all output

pcifind 1217 6836 1     ; find O2Micro 6836 cardbus controller, function 1

pciwritew 44 3E0        ; these commands will not execute if the

pciorw 4 7              ;   previous pcifind failed.

ob 3e0 0

ibfind 3e1 83           ; is pcmcia controller at 3e0?

ob 3e0 02

orb 3e1 10              ; set bit 4 of pcmcia register 2

show                    ; A little less output

pcifind 8086 122D 0     # dump registers for this device

pcireadd 0

pcireadd 4

pcireadd 8

pcireadd C

pcireadd 10

pcireadd 14

pcireadd 18

pcireadd 1C

pcireadd 20

pcireadd 24

pcireadd 28

pcireadd 2C

pause                   # stop to read the data on screen

pcifind FFFF FFFF 0     # this special pcifind will always be 'found'

OB 3f8 4a               # these commands will always happen.

OB 3f8 61

OB 3f8 79

ob 3f8 0d

ob 3f8 0a